After School Club Instructor’s Lesson Plan

Cat Apron

Materials:

2 x 14” x 18” Pieces of cotton for main apron

4 x pocket pieces

8 x ear pieces

2 x hearts for nose

4 x buttons for eyes

1 x 72” twill tape (1 ¼”)

Embroidery floss and hand sewing needles

- Pockets – with RST pin and sew around two sides, leaving the top open (1/4  - ½” seam)

- Clip curves

- Flip to right side

- Iron flat

- Turn in top edge and iron (1/4 – ½”)

- Hand stitch nose, mouth and eyes

- Ears - with RST pin and sew around two sides, leaving the top open (1/4  - ½” seam)

- Clip curves

- Flip to right side

- Iron flat

- Insert ears in to top of pocket

- Pin in place and stitch along top edge of pocket

- Pin pockets on apron

- Stitch sides, leaving top open

- Hand stitch whiskers

- Apron – With RST pin apron main pieces together (1/2” seam)

- Stitch leaving a 6” gap at top edge for turning

- Clip corners

- Flip to right side

- Iron

- Top stitch around sides and bottom

- Center twill tape on top edge of apron and pin in place

- Stitch

- Hem ends of twill tape

You are finished!