Quilted Tote Bag Week 3

EZ Strips

Additional Materials Required:

5 x 9” Fabric Strips Per Student

8” x 8” Lining Fabric

18” Rulers

Fabric Markers or Frixion Markers

Sticky Backed Fabric Squares

Iron & Ironing Board

Class

- Set-up, Arrival, Welcome & Safety (see separate info)

- Refresher of “Basic Anatomy of Sewing Machine” (5 Minutes)

    - On/Off Switch

    - Bobbin

    - Thread

    - Needle

    - Presser Foot

    - Hand/Fly Wheel

    - Presser Foot Lever

    - Foot Pedal

- Begin The EZ Strips

- Demonstrate how to stitch the strips of fabric together.

  - Choose 2 strips and place them on table, side by side and right sides up

  - Lift one piece up and place it face down (right sides together) on other piece of fabric. The edges should match all the way down

  - Draw a line 1/2” in from long edge of fabric all the way from top to bottom

  - Pin along the line leaving 4 fingers in between each pin. Make sure pins are pointing upwards

  - Insert fabric in to machine with needle inserted on the line almost at the top of the fabric

  - Stitch forward 3 stitches

  - Stitch backwards 3 stitches

  - Stitch all the way along the line until the needle reaches the end of the fabric

  - Stitch backwards 3 stitches

  - Stitch forwards 3 stitches

  - Raise the needle

  - Raise the presser foot

  - Pull the fabric out to the left and cut the thread on the cutter

  - Open out the fabric so both pieces are showing

  - Finger press the seam open.

  - Iron

- Add another strip in exactly the same way

- Continue to add strips until strips have been used. Try to keep the strips in a random order and start and end with 2 different fabrics. This will create a better patchwork effect.

- With right sides together stitch EZ Strip fabric to lining fabric along TOP EDGE ONLY

- Flip to right side and press seam flat

- Put aside until later